QUARC

Post-Quantum Secure Element

Product Requirements Document  //  v1.1  //  2025

"Quarc is a post-quantum Secure Element with hardware-accelerated ML-KEM and ML-DSA,

securing IoT devices today and anchoring trust for TEEs tomorrow."

| Field | Value |
| --- | --- |
| Document | Quarc PRD v1.1 |
| Status | DRAFT — Internal Only |
| Classification | Proprietary & Confidential |
| FPGA Board | ULX3S — Lattice ECP5-85K (84K LUTs) |
| CPU | Ibex RV32IMC (Apache 2.0, via sv2v + Yosys) |
| HDL Language | Verilog 2005 (all Quarc RTL) + sv2v for Ibex only |
| Toolchain | Yosys + nextpnr + sv2v + SymbiYosys + Icarus — 100% open source |
| Build System | GNU Make — single Makefile |
| IP License Model | Commercial closed IP — permissive OSS only (MIT/ISC/BSD/Apache 2.0) |
| PQC Standards | FIPS 203 ML-KEM-768 ¦ FIPS 204 ML-DSA-65 |
| v1.1 Changes | PMP policy table, Hardware KUE, Device lifecycle model, Anti-rollback, NTT zeroization, Entropy budgeting, Bus protocol, Concrete use case |

# 1.  Product Overview

## 1.1  Background and Motivation

Every Secure Element shipping today — Infineon SLx9, NXP SE050, Microchip ATECC608, and TROPIC01 — relies entirely on classical cryptography: ECC, RSA, and AES. These algorithms are broken by Shor's algorithm running on a cryptographically relevant quantum computer (CRQC).

NIST finalised its Post-Quantum Cryptography standards in 2024: ML-KEM (FIPS 203), ML-DSA (FIPS 204), and SLH-DSA (FIPS 205). Migration timelines from NIST, ENISA (EU), and BSI (Germany) project mandatory PQC adoption in critical infrastructure by 2030. IoT devices deployed today have operational lifetimes exceeding that window.

No shipping SE product today provides hardware-accelerated PQC. Quarc fills this gap. TROPIC01 by Tropic Square proves the market; Quarc extends it into the post-quantum era.

## 1.2  What Quarc Is

Quarc is a post-quantum Secure Element: a cryptographic coprocessor that integrates with a host MCU via SPI, provides hardware-accelerated PQC, securely stores device keys with hardware-enforced usage policy, and serves as the hardware root of trust.

## 1.3  What Quarc Is Not

- Not a clone of TROPIC01 — different crypto subsystem, same market positioning

- Not open-source — closed commercial IP built on permissive-licensed open components

- Not OpenTitan — deliberately lean, no SystemVerilog, no Bazel, fully open toolchain

- Not a crypto accelerator — Quarc enforces key usage in hardware; firmware never sees key bytes

- Not a finished product in v1 — v1 is an FPGA prototype proving architecture and PQC correctness

## 1.4  Concrete v1 Use Case: PQC Firmware OTA

PQC-authenticated firmware OTA update for an IoT gateway. This exercises every v1 capability end-to-end on the ULX3S with the onboard ESP32 as host — no external hardware required.

1. Factory:  Quarc generates ML-DSA keypair in slot 0 (device identity, SIGN_ONLY)

2. Factory:  Quarc vk registered with update server

3. Field:    Server generates new firmware, signs with its ML-DSA key

4. Field:    Host initiates Quarc session (CHANNEL_INIT — Noise IK + ML-KEM hybrid)

5. Field:    Host sends firmware + server signature (FW_UPDATE command)

6. Field:    Quarc verifies signature via ML-DSA + checks rollback counter

7. Field:    Quarc flashes firmware, increments rollback counter

8. Field:    Host requests attestation (DSA_SIGN over device state)

9. Field:    Server verifies attestation using registered vk

## 1.5  Competitive Positioning

| Feature | TROPIC01 | OpenTitan | ATECC608 | Quarc v1 |
| --- | --- | --- | --- | --- |
| CPU | Ibex (SV) | Ibex (SV) | None | Ibex RV32IMC (SV+sv2v) |
| Open toolchain | No | No | N/A | Yes — Yosys/nextpnr/sv2v |
| PQC hardware | None | None | None | ML-KEM-768 + ML-DSA-65 |
| HW key usage policy | Yes | Yes | Yes | Yes — KUE RTL |
| Device lifecycle | Yes | Yes | Partial | Yes — RTL FSM |
| Anti-rollback | Yes | Yes | Yes | Yes — Boot ROM |
| PMP — Boot ROM config | Yes | Yes | N/A | Yes — before firmware |
| Tamper resistance | Full (ASIC) | Full (ASIC) | Full (ASIC) | Stub hooks (v1 FPGA) |
| TEE RoT roadmap | No | Yes | No | Yes (v2+) |

# 2.  Objectives

## 2.1  Primary Objectives (v1)

- Hardware-accelerated ML-KEM-768: keygen, encapsulation, decapsulation

- Hardware-accelerated ML-DSA-65: keygen, sign, verify

- Shared NTT engine reused across both PQC algorithms

- Keccak/SHA-3/SHAKE engine as shared cryptographic primitive foundation

- TRNG with mandatory SP 800-90B health tests; hard halt on failure

- SHAKE-256 DRBG with entropy budgeting per operation

- Hardware Key Usage Enforcer (KUE) — crypto engines DMA keys directly; firmware never sees key bytes

- PMP configured and locked by Boot ROM before firmware executes — explicit region table

- Device lifecycle state machine: MANUFACTURING → PROVISIONED → LOCKED → RMA

- Command authorisation enforced per lifecycle state in RTL

- ML-DSA signed firmware boot with anti-rollback monotonic counter (Boot ROM enforced)

- NTT scratchpad zeroization after every PQC operation — RTL enforced, gates OP_DONE

- SPI host interface with Noise Protocol encrypted channel (hybrid ML-KEM + X25519)

- Ibex RV32IMC control plane — bare metal C, no OS

- 100% open-source toolchain: Yosys, nextpnr, sv2v, SymbiYosys, Icarus, openFPGALoader

- ULX3S ECP5-85K target platform

- All PQC accelerators validated against NIST KATs

## 2.2  Secondary Objectives (v1)

- Achieve >= 3x speedup vs pqm4 baseline on Cortex-M4

- Hybrid PQC + classical mode: ML-KEM + X25519, ML-DSA + Ed25519

- Host SDK in C for ESP32 and STM32

- SymbiYosys formal proofs: bus decoder, Keccak, PMP config, KUE policy, lifecycle FSM

- SoC fits < 15,000 LUTs on ECP5-85K

- >= 50 MHz clock frequency

- Complete v1 use case demo on ULX3S + ESP32

## 2.3  Explicitly Out of Scope (v1)

- Production-grade PUF — v2

- Physical tamper detection — FPGA cannot implement these

- Side-channel masking — v2 (constant-time and no secret-dependent addressing required in v1)

- Common Criteria or FIPS 140 certification

- ASIC implementation

- SLH-DSA — v2

- TEE attestation protocol — v2

- Real OTP / NVM — BRAM simulated in v1

- Fault injection resistance — v2 (invalid input fuzzing in v1 test plan)

# 3.  Threat Model

Every security requirement in this PRD is grounded in this threat model. A requirement without a corresponding threat is a feature, not a security requirement.

## 3.1  Protected Assets

- A1 — Device private keys: ML-DSA signing key, ML-KEM decapsulation key

- A2 — Device identity: X.509 certificate, attestation binding

- A3 — Session secrets: symmetric keys derived from key exchange

- A4 — Firmware integrity: authenticated boot image

- A5 — Entropy: TRNG output and DRBG state

- A6 — Device lifecycle state: MANUFACTURING / PROVISIONED / LOCKED / RMA

## 3.2  Threat Actors

| Actor | Capability | Goal | v1 Scope |
| --- | --- | --- | --- |
| Remote attacker | Software exploits via SPI API | Extract keys via firmware vuln | In scope — primary threat |
| Malicious host firmware | Arbitrary SPI commands | Abuse SE API beyond authorization | In scope — lifecycle + KUE enforcement |
| Quantum attacker | CRQC with Shor's algorithm | Break classical key exchange | In scope — PQC primary motivation |
| Compromised firmware | Firmware exploited post-boot | Read key bytes from registers | In scope — KUE + PMP block this |
| Passive physical | SPI sniffing, power trace | Key recovery from channel/power | Channel encrypted; power analysis v2 |
| Active physical | Fault injection, voltage glitch | Bypass security checks | Out of scope — v2+ |
| Supply chain | Malicious component insertion | Hardware backdoor | Architectural only in v1 |

## 3.3  Trust Boundary

On the ULX3S FPGA prototype, the trust boundary is a logical RTL boundary — not a physical one. This is an acknowledged v1 limitation.

Inside the trust boundary:

- Ibex CPU execution context and firmware memory (PMP-protected, configured by Boot ROM)

- Crypto subsystem: Keccak, NTT, ML-KEM, ML-DSA, TRNG, DRBG

- Key store BRAM (accessible only to crypto engines via internal DMA — not firmware)

- Key Usage Enforcer (KUE) — policy engine between key store and crypto engines

- Bus address decoder (formally verified)

- Boot ROM (immutable — synthesis-time Verilog ROM)

Outside the trust boundary — treated as untrusted:

- SPI bus and host MCU — all communication encrypted and authenticated

- External QSPI Flash — firmware signature-verified and rollback-checked before execution

- FPGA JTAG port — disabled in production bitstream

- UART debug — disabled in production bitstream

- Firmware itself — firmware orchestrates but never reads raw key bytes

## 3.4  Security Properties Required

- Confidentiality — private key bytes never accessible to firmware or SPI; KUE + PMP enforce in RTL

- Integrity — firmware ML-DSA verified; rollback counter enforced; no bypass possible

- Authenticity — device identity cryptographically bound to hardware-generated key

- Forward secrecy — host channel uses ephemeral key exchange per session

- Quantum resistance — all new crypto operations use NIST PQC standards

- Entropy assurance — TRNG health-tested; entropy budgeted per operation; failure halts

- Fail-secure — any fault causes halt and error assertion, not silent degradation

- Lifecycle integrity — state transitions are one-way, RTL-enforced, tamper-evident

# 4.  System Architecture

## 4.1  Design Principles

| ID | Principle | Detail |
| --- | --- | --- |
| P1 | Crypto-first, CPU-second | Ibex orchestrates; it does not do crypto. All heavy math is in hardware accelerators. |
| P2 | Lean by design | Every module justifies its presence. Complexity is a security liability. |
| P3 | Verilog 2005 only | No SystemVerilog in Quarc RTL. Ibex is the only SV component; sv2v converts it. |
| P4 | Open-source toolchain | Yosys, nextpnr, sv2v, SymbiYosys, Icarus. No proprietary EDA tools ever. |
| P5 | FPGA-first, ASIC-ready | No FPGA primitives in core modules. ASIC port requires no RTL rewrite. |
| P6 | Own critical IP | Entropy, PQC engines, bus, key store, KUE — proprietary Verilog. |
| P7 | Correctness before integration | 100% NIST KAT pass before SoC integration. No exceptions. |
| P8 | Permissive licenses only | MIT, ISC, BSD, Apache 2.0 only. Zero GPL or LGPL in synthesizable path. |
| P9 | Hardware enforces policy | Key usage, memory isolation, lifecycle state are RTL invariants. Compromised firmware cannot escalate privileges. |

## 4.2  SoC Architecture

Host MCU (STM32, ESP32, any)

| SPI — Noise Protocol IK (ML-KEM + X25519 hybrid, AES-256-GCM)

v

+-------------------------------------------------------------+

|                  QUARC SoC (ECP5-85K)                      |

|                                                             |

|  Boot ROM (immutable Verilog ROM)                          |

|    TRNG test → PMP configure+lock → ML-DSA verify fw       |

|    rollback check → jump to firmware                       |

|                                                             |

|  Ibex RV32IMC <──> Wishbone-lite bus (~200 lines Verilog)  |

|       |                                                     |

|  [Crypto Engine]    [Memory]         [Peripherals]         |

|  Keccak/SHAKE       IRAM (BRAM)       SPI slave            |

|  NTT engine         DRAM (BRAM)       UART (dev only)      |

|  ML-KEM ctrl        Key store ◄─ DMA  Timer/WDT            |

|  ML-DSA ctrl        (BRAM, no FW      Lifecycle reg        |

|  KUE (policy)        access via bus)  Rollback counter     |

|  TRNG + DRBG                                               |

+-------------------------------------------------------------+

Key store access: firmware issues KUE commands only.

KUE DMAs key bytes internally. Key bytes never on main bus.

## 4.3  Bus Architecture

Single-master Wishbone-lite bus. Ibex is the only bus master. Signals: cyc, stb, we, adr[31:0], dat_w[31:0], dat_r[31:0], ack, err. Handshake: transaction completes when stb & ack. Slaves assert ack when ready; err on unmapped access or policy violation.

DMA for crypto engines is internal to the crypto subsystem — not via the main bus. Ibex writes a control register; the engine signals completion via interrupt after operation and zeroization complete.

## 4.4  Module Inventory and LUT Budget

| Module | Est. LUTs | Source | Notes |
| --- | --- | --- | --- |
| Ibex RV32IMC | ~3,500 | Apache 2.0 | Control plane only. Synthesised via sv2v. |
| System Bus | ~200 | Proprietary | Wishbone-lite. Formally verified. |
| Keccak/SHAKE engine | ~1,500 | Proprietary | Shared. Built and verified first. |
| NTT engine (shared) | ~3,000 | Proprietary | Parameterised q=3329 and q=8380417. Zeroization RTL included. |
| ML-KEM-768 controller | ~1,500 | Proprietary | State machine. Drives NTT and Keccak. |
| ML-DSA-65 controller | ~2,000 | Proprietary | State machine. HW rejection sampler. Drives NTT. |
| Key Usage Enforcer (KUE) | ~200 | Proprietary | HW policy engine. SIGN/DECAP/NO_EXPORT per slot. Blocks firmware from key bytes. |
| TRNG | ~300 | Proprietary | Ring oscillator. SP 800-90B RCT + APT. |
| DRBG (SHAKE-256) | ~200 | Proprietary | Auto-reseed. Entropy budget counter. |
| SPI slave | ~400 | Proprietary | Mode 0/3. Command framing. 10 MHz. |
| Key store controller | ~200 + BRAM | Proprietary | BRAM-backed. DMA-only access from KUE. |
| Boot ROM | ~100 + BRAM | Proprietary | Verilog ROM. PMP config. ML-DSA root key. Rollback counter logic. |
| Lifecycle FSM | ~150 | Proprietary | 4-state. One-way transitions. RTL enforced. |
| Rollback counter | ~100 | Proprietary | Monotonic. BRAM-backed (v1). OTP in v2. |
| UART + Timer + WDT | ~350 | OSS / Proprietary | UART disabled in production. 100ms watchdog. |
| IRAM / DRAM | BRAM only | Inferred | No FPGA primitives. |
| TOTAL (estimate) | ~13,700 LUT |  | ~16% of ECP5-85K. Comfortable headroom. |

# 5.  Security Policy

This section defines policies enforced in hardware. These are RTL-level invariants. Firmware cannot override them.

## 5.1  PMP Region Layout

The Boot ROM configures and locks all PMP regions before jumping to firmware. Firmware cannot reconfigure PMP after boot. A compromised firmware cannot escape its sandbox.

| Region | Content | Permissions | Locked By | Notes |
| --- | --- | --- | --- | --- |
| 0 | Boot ROM | R-X | Boot ROM itself | Immutable. Firmware cannot write or re-execute. |
| 1 | Firmware IRAM | R-X | Boot ROM | Execute-only for firmware. No self-modification. |
| 2 | Data RAM | RW | Boot ROM | Firmware stack and heap. No execute. |
| 3 | Key store | --- (NO ACCESS) | Boot ROM | No firmware access. Crypto engines via internal DMA only. |
| 4 | Crypto MMIO | RW | Boot ROM | Control registers only. Key bytes never appear here. |
| 5 | SPI / UART / GPIO | RW | Boot ROM | Peripheral access. |
| 6 | Lifecycle register | RW (restricted) | Boot ROM | Write only accepted per lifecycle state. |
| 7 | Rollback counter | R only | Boot ROM | Firmware can read. Only Boot ROM increments. |

Boot sequence:

Power on

→ TRNG health test (halt if fail)

→ Configure PMP regions 0–7

→ Set PMP LOCK bits on all regions

→ Load firmware from QSPI Flash into IRAM

→ ML-DSA.Verify(root_vk, fw_hash, fw_sig)  [halt if fail]

→ Check fw_version >= rollback_counter       [halt if fail]

→ Jump to firmware entry point

## 5.2  Hardware Key Usage Policy (KUE)

The Key Usage Enforcer (KUE) sits between the key store and the crypto engines. Firmware issues commands to KUE — it never reads key bytes. KUE validates policy, DMAs the key internally, runs the operation, and zeroises engine key registers after completion.

| Policy Bit | Meaning | Enforced By |
| --- | --- | --- |
| SIGN_ONLY | Slot usable for ML-DSA signing only | KUE RTL — rejects DECAP or EXPORT requests |
| DECAP_ONLY | Slot usable for ML-KEM decap only | KUE RTL — rejects SIGN or EXPORT requests |
| NO_EXPORT | Key bytes never leave key store | KUE RTL — key bytes never appear on main bus |
| NO_OVERWRITE | Slot cannot be overwritten once provisioned | KUE RTL — rejects STORE_KEY to occupied slot |
| COUNTER_LIMIT | Maximum use count (16-bit) | KUE RTL — blocks operation when limit reached |

## 5.3  Device Lifecycle Model

Implemented in RTL. State transitions are one-way. No mechanism exists to return to a previous state.

MANUFACTURING ──→ PROVISIONED ──→ LOCKED

│                               │

└──────────────────────────────→ RMA

| Command | MFG | PROV | LOCK | RMA | Notes |
| --- | --- | --- | --- | --- | --- |
| PING / GET_STATUS | ✓ | ✓ | ✓ | ✓ | All states |
| GET_RANDOM | ✓ | ✓ | ✓ | ✗ |  |
| KEM/DSA KEYGEN | ✓ | ✓ | ✗ | ✗ | No key gen in LOCKED |
| KEM_ENCAP/DECAP | ✗ | ✓ | ✓ | ✗ | Not in MFG |
| DSA_SIGN | ✗ | ✓ | ✓ | ✗ | Not in MFG |
| DSA_VERIFY | ✓ | ✓ | ✓ | ✗ |  |
| STORE_KEY | ✓ | ✓ | ✗ | ✗ | No import in LOCKED |
| FW_UPDATE | ✓ | ✓ | ✗ | ✗ | No update in LOCKED |
| SECURE_ERASE | ✗ | ✓ | ✓ | ✓ | All except MFG |
| CHANNEL_INIT | ✓ | ✓ | ✓ | ✗ |  |
| SET_LIFECYCLE | →PROV | →LOCK/RMA | →RMA | ✗ | One-way only |

## 5.4  Secure Boot Anti-Rollback

The rollback counter is a 32-bit monotonic counter in Boot ROM address space. Firmware cannot write to it (PMP Region 7 — R only). Boot ROM reads and increments. Counter can only increase.

Firmware header format (signature covers full image including header):

[0:3]   magic      = 0x51415243  ('QARC')

[4:7]   fw_version = monotonic integer, must be >= rollback_counter

[8:11]  fw_length  = image length in bytes

[12:31] reserved

[32..]  firmware image

[end-2420..end]  ML-DSA-65 signature (2420 bytes)

## 5.5  NTT Scratchpad Zeroization

After every PQC operation, the NTT controller zeroes the full scratchpad BRAM before asserting OP_DONE. The interrupt fires only after zeroization completes. Firmware cannot skip or race this.

PQC operation completes

→ NTT asserts ZEROIZE

→ Counter walks all scratchpad addresses: write 0x00000000

→ All addresses zeroed: assert OP_DONE interrupt to Ibex

→ Firmware may now issue next command

## 5.6  Entropy Budget

| Operation | Entropy consumed | Notes |
| --- | --- | --- |
| ML-KEM-768 KeyGen | 64 bytes | DRBG seeded from TRNG |
| ML-KEM-768 Encaps | 32 bytes |  |
| ML-DSA-65 KeyGen | 32 bytes |  |
| ML-DSA-65 Sign | 32 bytes | rho' randomisation |
| Noise IK session | 32 bytes | Ephemeral key exchange |
| DRBG reseed policy | Every 1000 requests or 10 min | Halt if TRNG unavailable during required reseed |
| Minimum pool size | 256 bits | Required before any PQC operation permitted |

# 6.  Functional Requirements

## 6.1  ML-KEM Engine — FIPS 203, ML-KEM-768

- KeyGen, Encaps, Decaps — all hardware-accelerated

- Decapsulation key stored with DECAP_ONLY | NO_EXPORT policy — KUE enforced

- NTT, inverse NTT, Barrett reduction, SHAKE-128/256, rejection sampler, CBD sampler

- No secret-dependent memory addressing

- NTT scratchpad zeroed after every operation before OP_DONE

- 100% NIST FIPS 203 KAT pass before hardware integration

## 6.2  ML-DSA Engine — FIPS 204, ML-DSA-65

- KeyGen, Sign, Verify — all hardware-accelerated

- Signing key stored with SIGN_ONLY | NO_EXPORT policy — KUE enforced

- NTT, inverse NTT, hardware rejection sampling loop (not firmware), SHAKE-256, Power2Round, MakeHint

- No secret-dependent memory addressing

- NTT scratchpad zeroed after every operation before OP_DONE

- 100% NIST FIPS 204 KAT pass before hardware integration

## 6.3  Shared NTT Engine

- Parameterised: q=3329 (ML-KEM) and q=8380417 (ML-DSA) — selected via control register

- Iterative butterfly — area-efficient, constant and deterministic latency

- Constant-time — no data-dependent branching, no secret-dependent memory addressing

- Zeroization RTL — scratchpad zeroed before OP_DONE (Section 5.5)

- SymbiYosys formally verified before integration

## 6.4  Keccak / SHA-3 / SHAKE Engine

- Keccak-f[1600] — single shared instance

- SHA3-256, SHA3-512, SHAKE-128, SHAKE-256 modes

- Streaming interface — arbitrary-length absorb and squeeze

- 100% NIST SHA-3 KAT pass before integration

## 6.5  Entropy System

- TRNG: minimum 4 ring oscillators XOR'd; SP 800-90B RCT + APT in RTL; H_min >= 0.5 bits/sample

- Startup: device blocks all operations until entropy pool reaches 256-bit minimum and passes health tests

- TRNG failure: hard halt — no fallback

- DRBG: SHAKE-256 per SP 800-90A; reseeded per Section 5.6 policy; halt if reseed needed but TRNG unavailable

- Entropy budget tracked per operation type (Section 5.6)

## 6.6  Secure Key Storage

- 8 key slots minimum; each holds: key material, type tag, KUE policy flags, monotonic use counter

- No access from firmware or SPI — crypto engines only via internal DMA; PMP Region 3 + bus decoder enforce

- Key hierarchy: DRK in slot 0 (SIGN_ONLY | NO_EXPORT | NO_OVERWRITE)

- BRAM-backed in v1 — keys lost on power cycle; acknowledged limitation

## 6.7  Host Interface

- SPI slave — mode 0 and mode 3; 10 MHz max (v1); CS# active low; command/response model

- Noise Protocol IK: ML-KEM-768 + X25519 hybrid; AES-256-GCM channel; HMAC-SHA-256 MAC; 64-bit monotonic nonce

- All 14 commands lifecycle-gated per Section 5.3; KUE policy enforced per Section 5.2

## 6.8  Secure Boot

- Boot ROM configures and locks PMP before loading firmware (Section 5.1)

- TRNG health test → PMP lock → ML-DSA verify → rollback check → jump

- On any failure: halt unconditionally — no fallback, no retry

## 6.9  Firmware

- Ibex RV32IMC — bare metal C, no OS; riscv32-unknown-elf-gcc march=rv32imc -Os

- Firmware never reads key bytes — issues KUE commands only

- Watchdog serviced every 100ms — hang causes hardware reset

- Size target: < 64KB

# 7.  Non-Functional Requirements

## 7.1  Performance Targets

| Operation | Quarc target @ 50 MHz | pqm4 (M4 @ 168 MHz) | Min speedup |
| --- | --- | --- | --- |
| ML-KEM-768 KeyGen | < 0.5 ms | ~1.5 ms | >= 3x |
| ML-KEM-768 Encaps | < 0.5 ms | ~1.7 ms | >= 3x |
| ML-KEM-768 Decaps | < 0.5 ms | ~1.8 ms | >= 3x |
| ML-DSA-65 KeyGen | < 2 ms | ~6 ms | >= 3x |
| ML-DSA-65 Sign | < 5 ms | ~12 ms | >= 2x |
| ML-DSA-65 Verify | < 2 ms | ~5 ms | >= 2x |
| Keccak 1 block | < 0.1 ms | ~0.3 ms | >= 3x |
| NTT zeroization | < 0.1 ms | N/A | Must not dominate |

## 7.2  Resource Budget

- Total LUT: < 15,000 (< 18% of ECP5-85K)

- EBR BRAM: < 50% of 3744 Kbits

- Fmax: >= 50 MHz, timing enforced, no relaxation

## 7.3  Security Requirements

- No plaintext key bytes on main bus at any time

- No plaintext key bytes accessible to firmware — KUE RTL enforces

- Key store inaccessible from firmware — PMP Region 3 + bus decoder

- Bus decoder formally verified: no unmapped access; no key store access from firmware context

- TRNG failure: hard halt — no PRNG fallback

- Watchdog: hard reset if not serviced within 100ms

- JTAG and UART disabled in production bitstream

- Constant-time throughout crypto path — no data-dependent branching

- No secret-dependent memory addressing anywhere

- NTT scratchpad zeroed after every operation — RTL enforced, gates OP_DONE

## 7.4  Auditability

- One Verilog module per file; max 500 lines per module

- Header comment on every module: purpose, interfaces, assumptions

- No FPGA-specific primitives in core logic — inferred only

- No initial blocks, $display, $finish in synthesizable RTL

- All constants named with parameter; module hierarchy max 4 levels

## 7.5  Toolchain and Build

- 100% open-source synthesis, P&R, simulation, formal — no proprietary EDA tools

- GNU Make — single Makefile, reproducible, no external build dependencies

- Clean clone + make all produces identical bitstream on any Linux host

# 8.  Key Design Decisions

These decisions are locked for v1. Revisiting them requires a formal PRD amendment.

| Decision | Chosen | Rejected | Rationale |
| --- | --- | --- | --- |
| HDL language | Verilog 2005 (Quarc RTL) | SystemVerilog | Yosys native. sv2v handles Ibex only. |
| CPU core | Ibex RV32IMC (sv2v) | PicoRV32 (no PMP) | Hardware PMP mandatory — Boot ROM must lock regions before firmware. SE-proven. |
| Key access model | HW DMA only (KUE) | Firmware-readable registers | Firmware compromise cannot expose key bytes. This is the SE/accelerator distinction. |
| PMP configuration | Boot ROM locks before jump | Firmware configures | Compromised firmware cannot reconfigure memory isolation. |
| Lifecycle enforcement | RTL state machine | Firmware flag | Hardware lifecycle cannot be bypassed by compromised firmware. |
| Rollback protection | Boot ROM counter | Firmware check | Firmware cannot skip its own downgrade check. |
| NTT zeroization | RTL enforced, blocks OP_DONE | Firmware responsibility | Firmware cannot forget. Timing side-channel closed in hardware. |
| FPGA family | Lattice ECP5 | Xilinx Artix-7 | Full open-source P&R. Xilinx requires Vivado. |
| Bus | Custom Wishbone-lite | AXI4, TileLink | ~200 lines, formally verifiable, single-master, fully owned. |
| Host channel | Noise IK + hybrid KEM | TLS 1.3, custom | Proven SE pattern. Hybrid = quantum resistance + classical compat. |
| Firmware environment | Bare metal C | FreeRTOS, Zephyr | Minimum attack surface. Deterministic timing. |
| NTT architecture | Iterative butterfly | Parallel / unrolled | Area-efficient. Formally verifiable. Constant latency. |

# 9.  Test and Verification Strategy

## 9.1  Verification Levels

- Module simulation — dedicated Icarus testbench per module

- Known Answer Tests — all crypto vs official NIST vectors

- Security policy simulation — KUE violations, PMP violations, lifecycle rejections, rollback attacks

- Invalid input fuzzing — malformed SPI commands, invalid slot references, boundary conditions

- Integration simulation — full SoC with software SPI master

- Formal verification — SymbiYosys on bus decoder, Keccak, TRNG, PMP config, KUE, lifecycle FSM

- Hardware-in-loop — physical ULX3S with onboard ESP32

## 9.2  KAT Requirements

| Module | KAT Source | Pass Criterion |
| --- | --- | --- |
| Keccak/SHA-3/SHAKE | NIST SHA-3 KAT | 100% all modes |
| ML-KEM-768 | NIST FIPS 203 KAT | 100% keygen, encaps, decaps |
| ML-DSA-65 | NIST FIPS 204 KAT | 100% keygen, sign, verify |
| AES-256-GCM | NIST CAVS / ACVTS | 100% encrypt and decrypt |
| DRBG (SHAKE-256) | NIST SP 800-90A KAT | 100% generate with reseed |
| TRNG health tests | Injected fault test | RCT and APT trigger halt within specification window |

## 9.3  Formal Verification Targets

- Bus decoder: no address maps to two slaves; no unmapped access returns data; key store inaccessible from firmware context

- Keccak engine: output matches reference for all inputs up to defined bound

- TRNG health logic: RCT and APT thresholds trigger halt correctly

- PMP configuration: all regions locked before jump-to-firmware asserted — Boot ROM invariant

- KUE policy engine: SIGN_ONLY slot rejected for DECAP; NO_EXPORT never emits key bytes to bus; COUNTER_LIMIT blocks at limit

- Lifecycle FSM: no backward transition; SET_LIFECYCLE only accepted in correct source states

## 9.4  Security Simulation Tests

- KUE policy violation: DECAP with SIGN_ONLY slot → expect ERR_KEY_POLICY

- Lifecycle violation: FW_UPDATE in LOCKED state → expect ERR_LIFECYCLE

- PMP violation: firmware reads key store address → expect bus err + halt

- Rollback attack: fw_version < rollback_counter → expect boot halt

- Invalid Noise handshake: tampered session key → expect channel drop

- TRNG failure injection: force RCT failure → expect device halt within health test window

- Zeroization verification: read NTT scratchpad after operation via debug interface → expect all zeros

## 9.5  v1 Success Criteria — All Must Pass

- ML-KEM-768 full round trip correct on hardware

- ML-DSA-65 full round trip correct on hardware

- 100% NIST KAT pass for Keccak, ML-KEM-768, ML-DSA-65, AES-256-GCM, DRBG

- Hardware randomness passes NIST SP 800-22

- SPI command protocol correct on all 14 commands across applicable lifecycle states

- Noise IK handshake, session, AES-GCM working

- Secure boot: unsigned rejected, signed accepted, rollback rejected

- KUE policy violations correctly rejected in simulation and hardware

- Lifecycle state transitions correct and irreversible

- NTT scratchpad verified zero after every operation

- PMP regions locked by Boot ROM before firmware entry (formal proof)

- Ibex firmware 24-hour soak without fault

- Total LUT < 15,000 on ECP5-85K

- Clock >= 50 MHz

- SymbiYosys proofs pass: bus decoder, Keccak, KUE, PMP config, lifecycle FSM

- v1 use case demo (Section 1.4) runs end-to-end on ULX3S + ESP32

# 10.  Development Phases

Each phase must pass its exit criteria before the next begins. No exceptions.

| Phase | Weeks | Deliverables | Exit Criteria |
| --- | --- | --- | --- |
| 0 — Foundation | 1–2 | Ibex (sv2v) on ECP5-85K. UART hello world. Memory map + PMP table defined. Bus skeleton. Boot ROM stub. | Ibex boots. UART correct. Bitstream programs. PMP table in spec. |
| 1 — Keccak | 3–5 | Keccak/SHAKE RTL. Icarus testbench. NIST KAT runner. SymbiYosys formal. | 100% NIST SHA-3 KAT. Formal complete. Accessible from Ibex. |
| 2 — Entropy | 5–7 | TRNG RTL. SP 800-90B RCT + APT in RTL. SHAKE DRBG with entropy budget counter. | SP 800-22 pass. Health test halts on injected failure. Budget counter functional. |
| 3 — NTT Engine | 7–10 | NTT/iNTT iterative butterfly. q=3329. Scratchpad BRAM. Zeroization RTL. Constant-time. SymbiYosys formal. | NTT(iNTT(p))==p. Formally verified. Zeroization verified in sim. |
| 4 — ML-KEM | 10–14 | ML-KEM-768 controller. Keygen/Encaps/Decaps. KUE integration (DECAP_ONLY). Firmware driver. | 100% FIPS 203 KAT. KUE rejection confirmed. |
| 5 — ML-DSA | 13–17 | NTT reparameterised q=8380417. ML-DSA-65. HW rejection sampler. KUE (SIGN_ONLY). Firmware driver. | 100% FIPS 204 KAT. KUE rejection confirmed. |
| 6 — Security Policy | 16–20 | KUE full RTL. PMP Boot ROM config. Lifecycle FSM. Rollback counter. SymbiYosys formal on all policy modules. | All formal proofs pass. Section 9.4 security simulation tests pass. |
| 7 — SPI + Channel | 19–23 | SPI slave RTL. Noise IK + hybrid ML-KEM + X25519. AES-256-GCM. Host SDK in C. | Encrypted session. Forward secrecy. Lifecycle-gated commands working. |
| 8 — Integration | 22–26 | Secure boot ROM (PMP + ML-DSA + rollback). Full SoC integration. 24h soak. v1 use case demo. | All Section 9.5 criteria pass. Demo runs on ULX3S + ESP32. |

# 11.  Toolchain Reference

| Tool | Purpose | License | Notes |
| --- | --- | --- | --- |
| sv2v | SV to Verilog conversion | MIT | Converts Ibex SV to Verilog before Yosys. Single Makefile step, deterministic. |
| Yosys | RTL synthesis | ISC | synth_ecp5 -abc9. Native Verilog 2005. |
| nextpnr-ecp5 | Place and route | ISC | Project Trellis backend. Timing-driven. |
| ecppack | Bitstream generation | MIT | Part of Project Trellis. |
| openFPGALoader | FPGA programming | Apache 2.0 | Programs ULX3S natively. |
| Icarus Verilog | Functional simulation | GPL (sim only) | iverilog + vvp. GTKWave for waveforms. |
| Verilator | Fast regression simulation | LGPL (sim only) | C++ compiled. 10–100x faster than Icarus. |
| SymbiYosys | Formal verification | ISC | BMC and k-induction. All policy module targets. |
| riscv32-unknown-elf-gcc | Firmware compiler | GPL (compiler) | march=rv32imc. Bare metal. |
| GNU Make | Build orchestration | GPL (tool) | Single Makefile. make sim/synth/pnr/prog/formal. |
| OSS CAD Suite | All-in-one installer | Various | YosysHQ. Installs entire toolchain. |

GPL applies only to the tools themselves — not to RTL or firmware produced by them.

# 12.  Risks and Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| NTT area exceeds budget | Medium | High | Iterative architecture. 13.7K of 15K estimate. Scratchpad size tunable. |
| nextpnr timing failure | Medium | Medium | 50 MHz conservative for ECP5. Pipeline critical path if needed. -abc9. |
| TRNG entropy insufficient | Low | Critical | Conservative H_min. Multiple oscillators. SP 800-90B in RTL. Entropy budgeting enforced. |
| ML-DSA timing leak | Medium | High | Hardware rejection sampler loop. Constant iteration count. Simulation verified. |
| BRAM key store power cycle loss | Certain | Low (v1) | Acknowledged v1 limitation. OTP planned for v2. |
| KUE policy bypass via bus timing | Low | Critical | KUE formally verified. Bus err on policy violation. Single-master bus has no race condition. |
| PMP misconfiguration in Boot ROM | Low | Critical | Boot ROM is synthesis-time Verilog ROM — cannot be modified. PMP config formally verified (sby). |
| Rollback counter reset attack | Low | High | Counter in Boot ROM address space. PMP blocks firmware write. Increment is RTL logic. |
| Lifecycle FSM bypass | Low | High | FSM formally verified. State register in hardware — not firmware-writable memory. |
| Zeroization incomplete before OP_DONE | Low | Medium | OP_DONE gated on zeroization complete in RTL. Cannot be bypassed by firmware. |
| Noise Protocol implementation error | Medium | High | Reference test vectors. C firmware. Independent code review. |
| Scope creep | High | Medium | PRD is scope lock. Section 2.3 is the out-of-scope list. |

# 13.  Future Roadmap

v1 proves the architecture. Roadmap contingent on v1 success criteria passing.

## 13.1  v2 — Hardened SE

- PUF — device identity rooted in silicon entropy; fuzzy extractor

- Real OTP / NVM — persistent key storage

- Tamper detection: voltage glitch, temperature, EMP (ASIC)

- Active shield (ASIC only)

- Side-channel countermeasures: Boolean masking on NTT, randomised execution order

- Memory address scrambling and on-the-fly encryption for key store

- SLH-DSA hash-based signatures — fully quantum-safe without lattice assumptions

- Common Criteria EAL4+ certification path

## 13.2  v3 — TEE Root of Trust

- PQC attestation: ML-DSA-signed reports for confidential computing platforms

- Sealing keys: derive keys bound to firmware measurement (PCR-style)

- Secure provisioning: ML-KEM encrypted key injection

- Migration keys: quantum-safe key migration between devices

- Platform interface: PCIe or I3C for server/TEE integration

- Targets: AMD SEV-SNP, Intel TDX, Arm CCA

## 13.3  ASIC Path

- FPGA prototype validates RTL — ASIC port requires no core rewrite (P5)

- ASIC additions: standard cell library, DFT, physical design, tamper mesh

- Technology node: 40nm or 22nm FD-SOI

- Certification: FIPS 140-3 Level 3, Common Criteria EAL5+

# 14.  Glossary

| Term | Definition |
| --- | --- |
| CRQC | Cryptographically Relevant Quantum Computer — capable of running Shor's algorithm to break RSA and ECC |
| DRBG | Deterministic Random Bit Generator — cryptographic PRNG seeded from TRNG, per NIST SP 800-90A |
| DRK | Device Root Key — slot 0, SIGN_ONLY ¦ NO_EXPORT ¦ NO_OVERWRITE; device attestation anchor |
| EBR | Embedded Block RAM — dedicated SRAM blocks in Lattice ECP5 |
| ECP5 | Lattice Semiconductor ECP5 FPGA family — fully open-source toolchain (Yosys/nextpnr/Project Trellis) |
| FIPS 203 | US Federal Information Processing Standard 203 — specifies ML-KEM |
| FIPS 204 | US Federal Information Processing Standard 204 — specifies ML-DSA |
| Ibex | Small 32-bit RISC-V CPU by lowRISC (Apache 2.0). Quarc's control plane. Synthesised via sv2v. SE-proven in OpenTitan and TROPIC01. |
| KAT | Known Answer Test — NIST-published test vector for cryptographic validation |
| KUE | Key Usage Enforcer — Quarc RTL policy engine enforcing per-slot key usage. Firmware never reads key bytes; KUE DMAs internally. |
| ML-DSA | Module Lattice Digital Signature Algorithm (FIPS 204) — post-quantum signatures, also known as Dilithium |
| ML-KEM | Module Lattice Key Encapsulation Mechanism (FIPS 203) — post-quantum key exchange, also known as Kyber |
| NTT | Number Theoretic Transform — FFT analogue over finite field; core operation in lattice-based PQC |
| Noise Protocol | Framework for building secure channel protocols with formal security proofs. Pattern IK used for SE host channel. |
| PMP | Physical Memory Protection — RISC-V hardware extension in Ibex. Configured and locked by Boot ROM before firmware executes. |
| PQC | Post-Quantum Cryptography — algorithms believed to resist attacks by quantum computers |
| PUF | Physically Unclonable Function — circuit producing a unique response from manufacturing variations |
| RoT | Root of Trust — hardware component that all subsequent device security depends upon |
| SE | Secure Element — tamper-resistant microprocessor that stores secrets and performs crypto in isolation |
| SPI | Serial Peripheral Interface — synchronous serial protocol for host MCU to Quarc communication |
| sv2v | SystemVerilog-to-Verilog converter (MIT). Converts Ibex RTL for Yosys. Deterministic, open source. |
| TEE | Trusted Execution Environment — isolated compute environment (AMD SEV-SNP, Intel TDX, Arm CCA) |
| TRNG | True Random Number Generator — randomness from physical entropy source (ring oscillators in Quarc) |
| ULX3S | Open hardware FPGA board by Radiona.org featuring Lattice ECP5-85K and full open-source tool support |
| Wishbone | Open-source hardware bus standard. Quarc uses a simplified single-master Wishbone-compatible bus. |
| XOF | Extendable Output Function — hash variant producing arbitrary-length output (SHAKE-128, SHAKE-256) |

END OF DOCUMENT

Quarc Secure Element  //  PRD v1.1  //  2025  //  Proprietary & Confidential